# Testing Document

Introduction

This document describes the testing procedures used to verify the functionality, safety and reliability of this project. The moving arm integrates multiple servo motors, a microcontroller and inverse kinematics to perform controlled and coordinated motion. Because the system combines mechanical components, electrical hardware and software logic, structured testing is necessary to ensure that each subsystem operates correctly both individually and as part of the complete system.

Objective

The objective of this testing document is to define a set of manual and automatic tests that can be performed by any user or developer building the moving arm to confirm proper operation at the user level. These tests are designed to verify that:

- Each servo motor responds correctly and remains within safe mechanical limits

- Automatic and inverse-kinematics based movements behave predictably and smoothly

- The robotic arm performs coordinated motion across multiple degrees of freedom

- User feedback mechanisms (such as the LCD display and status indicators) function as intended

- The system operates safely without causing mechanical interference, overheating or damage

Successful completion of the tests outlined in this document demonstrates that the moving arm system meets functional requirements and is suitable for demonstration and further development.

- Test Environment & Pre-Checks

  - Hardware Setup

    - All servos are connected to the correct pins

    - External 5 V power is used for the servos

    - MPU6050 (if used) is wired correctly (SDA, SCL, VCC, GND)

    - LCD is connected to the correct pins

    - Receiver

    - Transmitter

  - Software Setup

    - The correct Arduino sketch is uploaded

    - Arduino Serial Monitor is set to the correct baud rate

  - Safety

    - The arm has enough free space without hitting objects

    - Nothing is blocking the joints

    - One hand is near the power switch in case of unexpected behavior

- Manual Tests (User-Level Verification): These tests can be done by anyone building the system to confirm basic functionality

  - Servo Power & Idle Test: Verify that all servos power on and hold position

    - Steps

      - Power on the system with the test code loaded

      - Confirm each servo moves briefly to a neutral position or holds still (depending on the sketch)

      - Lightly touch each joint to feel that it resists motion (servo is powered)

    - Pass Criteria:

      - All joints feel “stiff” (powered)

      - No servo makes continuous grinding and stalling noise

  - LCD Feedback Test: Verify that the LCD provides meaningful user feedback (e.g., servo angles or system status).

    - Steps

      - Power on the system

      - Confirm that the initial message appears (e.g., “Initializing…” or “System Ready!”)

      - Move the arm or let the automatic routine run

      - Confirm that the LCD updates with Base angle, Gripper angle or other configured values (e.g., pitch angle, status text).

    - Pass Criteria

      - All LCD characters are visible and readable

      - Display updates when servos move (no frozen display)

      - Values makes sense (e.g., angles in a reasonable range, not random numbers)

  - Automatic Tests (No Manual Input Required): These tests rely on the automatic demo code to verify system behavior at the user level

    - Steps

      - Upload the automatic 5-DoF test code with limits

      - Power the system

      - Observe the sequence

        - Base rotates from its minimum angle to maximum and back

        - Shoulder moves up and down

        - Elbow bends and strengthens

        - Wrist pitch tilts up and down

        - Gripper opens and closes within safe range

    - Pass Criteria

      - Each joint moves smoothly with no jerky movements

      - No joint moves beyond its mechanical limits

      - Gripper does not cross or jam when “closing”

      - System can complete several cycles without overheating or stalling

  - Gripper Safety Test: Verify that the gripper never fully crosses or damages itself

    - Steps

      - Run the automatic motion sequence

      - Watch only the gripper joint

      - Observe the fully open and fully closed positions

    - Pass Criteria

      - In “closed” position, the gripper fingers are gripping but not overlapping or forcing each other

      - No clicking, grinding, or visible strain on the plastic

      - Servo returns smoothly to open position

  - Basic Safety and Reliability Tests: Ensure the system can operate for a short continuous period without failure

    - Steps

      - Run the automatic demo motion for 5-10 minutes

      - Periodically touch the servos to check temperature

      - Listen for unusual noises or stalling

    - Pass Criteria

      - No servo gets dangerously hot

      - No mechanical parts come loose

      - Motions remain consistent over time

Summary

By performing the tests above, a user can verify that all motors are powered and mapped correctly, motion is smooth and stays inside safe limits, the LCD correctly reports system status or angles, the gripper operates safely and does not over-close. These tests together confirm that the major user-level functionality of the 5 DoF robotic arm is working as intended.